[image: image6.wmf][image: image7.wmf][image: image8.wmf]                     
[image: image9]            

[image: image10.wmf]                                        1              


[image: image11.jpg]GN‘O"R

Lighting
LED TECHNOLOGY



                                                                                                

                                                                                                                                 

5、产品最大绝对参数值（Ta：25(C）ABSOLUTE MAXIMUM RATINGS AT Ta=25(C：
	参数（PARAMETER）
	最大值Max.
	单位UNIT

	每段功耗 Power Dissipation Per Segment  
	52
	mW

	每段峰值电流 Peak Forward Current Per Segment  

(1/10duty cycle 0.1ms pulse width)
	100
	mA

	每段平均正向电流  Average Forward Current Per Segment 
	20
	mA

	从室温线性减少  Derating Linear From 25(C Per Segment 
	0.33
	mA/(C

	每段反向电压  Reverse Voltage Per Segment  


	5
	V

	工作条件温度  Operating Temperature Range  
	-40(C to + 105(C

	储存温度   Storage Temperature Range 
	-40(C to + 105(C

	距离胶体1.6mm焊接，温度260(C，焊接时间最多3秒

Lead Soldering Temperature 260(C at 1.6mm From Body for 3 seconds
 （


6、产品光电参数值（Ta：25(C）ELECTRICAL/OPTICAL CHARACTERISTICS AT Ta=25(C：

	参数  

PARAMETER
	符号

SYMBOL
	MIN.
	TYP.
	MAX.
	UNIT
	测试条件

Test condition

	每段亮度Luminous Intensity Per Segment
	Iv
	(
	​​​​​3
	(
	mcd
	IF=10mA

	主波长Dominant Wavelength 
	(D
	(
	640
	(
	nm
	IF=20mA

	光谱半宽度Spectral Line Half-Width  
	((
	(
	30
	(
	nm
	IF=20mA

	每颗正向电压Forward Voltage Per Dice
	VF
	(
	2.0
	2.6
	V
	IF=20mA

	每颗反向电流Reverse Current Per Dice
	IR
	(
	(
	50
	(  A
	VR =5V


7、推荐使用的焊盘尺寸Recommended Soldering Pattern：


8、电气/光学特性曲线Electrical / Optical Charateristics Curves:

(Ta = 25℃ Unless Otherwise Noted)


[image: image1.wmf]
9、推荐的SMT焊接曲线Recommended SMT condition：

[image: image2.wmf]0

50

100

150

200

250

300

50

100

150

200

250

300

（sec）�

Time

60~120s

30~50s

Temperture

25¡æ

(℃)

5 s max

4℃/s max�

4℃/s max�

4℃/s max�

150~180℃�

260℃ max�

230℃�

NOTES：�

1.We recommend the reflow temperature 245℃(+/-5℃).The maximum�

soldering temperature should be limited to 260℃.�

2.Don't cause stress to the epoxy resin while it is exposed to

high temperature.

3.Number of reflow process shall be 2 times or less.


10、卷轴规格  REEL DIMENSIONS：

[image: image3.wmf]
11、包装PACKING & LABEL SPECIFICATIONS


[image: image4.wmf]

 EMBED AutoCAD.Drawing.17  [image: image5.wmf]
产品外形尺寸Outer Dimension：


� EMBED AutoCAD.Drawing.17  ���


备注：未标注公差的尺寸均为±0.25mm


NOTES: All dimensions are in millimeters (inches) tolerance are (0.25mm(0.010) unless otherwise noted





1.产品描述DESCRIPTION：


灰色显示面，白色笔段 Gray face，White Epoxy


共阳显示屏 Common Anode Display


红色显示，Red Color


符合RoHS要求 RoHS Compliant








2.产品特征FEATURES：    


高可靠性和高稳定性 High intensity and reliability


高品质、和低功耗 High quality, Low power requirement 


IC易兼容，易装配  IC compatible , Easy assembly


ESD1000V











4. 产品线路图INTERNAL CIRCUIT DIAGRAM





� EMBED AutoCAD.Drawing.17  ���� EMBED AutoCAD.Drawing.17  ���





� EMBED  \* MERGEFORMAT ���








- 1 -

GNOR


_1234567895.dwg
49302


_1234567897.unknown

_1234567898.dwg
17901


_1234567896.dwg

_1234567891.dwg

_1234567893.dwg
Administrator


_1234567894.dwg
49302


_1234567892.dwg
Administrator


_1234567890.dwg
17901


