G-NOR ELECTRONICS CO.,LTD.

| GNL-1880xx (7.62x7.62) PIRANHA TYPE # Flux LED Lamp ABSOLUTE MAXIMUM RATING:(Ta=25℃) Parameter; Unit. DC Forward Current; 40; mA. Reverse Voltage; 5; V. Reverse Current (Vr-5V); 10; µA. Peak Forward Current ,tw ,=1 msec Duty ,=1/20; 180; mA. Operation Temperature; -40; +80; ℃. Storage Temperature; -40; +100; ℃. Solder Dip (5 seconds,1.6mm from body ) Temperature; 300; ±5; ℃. |  |
| --- | --- |
| # Part Number; EMITTED COLOR; Lens Color; Peak Wavelength (nm); Vf (v) Typ.; IV (If=20mA) mcd; View Angle 2θ1/2. Min.; Typ.. GNL-1880UEC; AlGaInP; Orange Red; Water Clear; 625; 2.0; 350; 450; 70°. GNL-1880URC; GaAlAs; Pure Red; Water Clear; 660; 1.9; 250; 350; 70°. GNL-1880UYC; AlGaInP; Yellow; Water Clear; 590; 2.0; 350; 450; 70°. GNL-1880UEC-TL; AlGaInP; Orange Red; Water Clear; 625; 2.0; 800; 1000; 70°. GNL-1880UYC-TL; AlGaInP; Yellow; Water Clear; 590; 2.0; 1100; 1300; 70°. GNL-1880UYOC-TL; AlGaInP; Amber; Water Clear; 610; 2.0; 1100; 1300; 70°. GNL-1880UBC; InGaN; Blue; Water Clear; 470; 3.5; 600; 800; 70°. GNL-1880BGC; InGaN; Cyan; Water Clear; 505; 3.5; 700; 900; 70°. GNL-1880PGC; InGaN; Pure Green; Water Clear; 525; 3.5; 1400; 1800; 70°. GNL-1880UBC-TL; InGaN; Blue; Water Clear; 470; 3.5; 800; 900; 70°. GNL-1880BGC-TL; InGaN; Cyan; Water Clear; 505; 3.5; 900; 1100; 70°. GNL-1880PGC-TL; InGaN; Pure Green; Water Clear; 525; 3.5; 3000; 4000; 70°. GNL-1880UWC; InGaN; Ultra White; Water Clear; 5500K-7000K; 3.5; 800; 1200; 70°. GNL-1880UWC-15C; InGaN; Ultra White; Water Clear; 5500K-7000K; 3.5; 1700; 1900; 70°. GNL-1880UWC-TL-15C; InGaN; Ultra White; Water Clear; 5500K-7000K; 3.5; 4000; 5000; 90°. |