G-NOR ELECTRONICS CO.,LTD.

| GNL-3015xx (Φ3mm ROUND TYPE) # Bi-color series ABSOLUTE MAXIMUM RATING:(Ta=25℃) Parameter; Unit. DC Forward Current; 30; mA. Reverse Voltage; 5; V. Reverse Current (Vr-5V); 10; µA. Peak Forward Current ,tw ,=1 msec Duty ,=1/20; 150; mA. Operation Temperature; -40; +80; ℃. Storage Temperature; -40; +100; ℃. Solder Dip Temperature (5 seconds,1.6mm from body ); 300; ±5; ℃. |  |
| --- | --- |
| Part Number; EMITTED COLOR; Mix Emitted Color; Lens Color; Peak Wave length (nm); Vf (v) Typ.; IV (If=20mA) mcd; View Angle 2θ1/2. Min.; Typ.. GNL-3015EGW; GaAsp/Gap; Hi-Eff Red; Orange; White Diffused; 635; 2.0; 8.0; 15; 60°. Gap; Green; 565; 2.1; 5.0; 10. GNL-3015YGW; Gap; Green; Yellow-Green; White Diffused; 565; 2.1; 5.0; 10; 60°. GaAsp; Yellow; 585; 2.1; 5.0; 10. GNL-3015UEUBW; AlGaInP; Super Red; Purple; White Diffused; 625; 2.0; 120; 200; 60°. InGaN; Super Blue; 470; 3.5; 110; 180. GNL-3015UEUGW; AlGaInP; Super Red; Orange; White Diffused; 625; 2.0; 120; 200; 60°. AlGaInP; Super Green; 572; 2.0; 100; 150. GNL-3015EGC; GaAsp/Gap; Hi-Eff Red; Orange; Water Clear; 635; 2.0; 20; 40; 30°. Gap; Green; 565; 2.1; 15; 25. GNL-3015UYUGC; AlGaInP; Super yellow; Yellow-Green; Water Clear; 590; 2.1; 270; 500; 30°. AlGaInP; Super Green; 572; 2.1; 260; 500. GNL-3015UEUGC; AlGaInP; Super Red; Orange; Water Clear; 625; 2.0; 280; 500; 30°. AlGaInP; Super Green; 572; 2.0; 260; 350. GNL-3015UEUBC; AlGaInP; Super Red; Purple; Water Clear; 625; 2.0; 280; 500; 30°. InGaN; Super Blue; 470; 3.5; 350; 450. |