PAGE1/5

SPECIFICATION

PRODUCT TYPE：  OF9745P-2A303

Engineering No.:

Customer Material Code：

Customer Type:

| # DSND BY |  |  |
| --- | --- | --- |
| CHKD BY |  |  |
| APRVD BY |  |  |

PAGE 2/5

| 1 | Name：Omnidirectional Electret Condenser Microphone (Foil Electret Type) |  |  |
| --- | --- | --- | --- |
| 2 | TYPE：OF9745P-2A303 |  |  |
| 3 | Electrical Specifications: |  |  |
|  | 3.1 | ## Sensitivity Range | -30±3dB RL=2.2KΩ Vs=4.5V ( 1KHz 0dB=1V/Pa) |
|  | 3.2 | Impedance | Max: 2.2KΩ 1KHz (RL=2.2KΩ) |
|  | 3.3 | Frequency | 50-16000Hz |
|  | 3.4 | Current Consumption | Max.0.5mA RL=2.2KΩ Vs=4.5V |
|  | 3.5 | Operation Voltage Range | 1.0V-10V(DC) |
|  | 3.6 | Max. Sound Pressure Level | More than 120dB S.P.L（1KHz，THD<3%） |
|  | 3.7 | S/N Ratio | More than 60dB (1KHz 0dB=1V/Pa，A Weighted) |
|  | 3.8 | Sensitivity Reduction | 3.0V-2.0V Sensitivity Variation less than 3dB |
|  | 3.9 Typical Frequency Response Curve：B&K2012 50cm |  |  |
|  | 3.10 Microphone Circuit Diagram： |  |  |

PAGE 3/5

|  | 3.11 Schematic Measuring Diagram： |  |  |
| --- | --- | --- | --- |
| 4 Mechanical Specifications： |  |  |  |
|  | 4.1 | ## Drawing |  |
|  | 4.2 | Dimension (mm): |  |
|  | 4.3 | ## Weight | 0.8g |
|  | 4.4 | ##### Mechanical Intensity | To be no interference in operation after pulled the terminals with 1.0Kg weight for 1 minute. |

PAGE 4/5

| 5.Reliability Tests: After any following tests, the sensitivity of the microphone unit shall not change more than ±3dB from initial value, and shall keep their initial operation and appearance. |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5.1 |  | ##### Hi-Temp. Test | The microphone unit must be subjected to +70℃ for 200 Hours, and expose to room temperature for 3 Hours. |
|  | 5.2 |  | Low-Temp. Test | The microphone unit must be subjected to -25℃ for 200 Hours, and expose to room temperature for 3 Hours. |
|  | 5.3 |  | Humi.&Heat Test | The microphone unit must be subjected to +60℃, 90% RH-for 200 Hours, and expose to room temp for 3 Hours . |
|  | 5.4 |  | Humidity Shocking Test | The microphone unit must be subjected to following conditions (+50℃ 1H-room temp 1H;-10℃ 1H-room temp 1H) at 5 cycle, and expose to room temp for 3 Hours. |
|  | 5.5 |  | Vibration Test | The microphone unit must be subjected to a procedure that after vibrating for two hours from each of the two directions with a frequency of 10-55Hz and a 1.52mm-high amplitude. |
|  | 5.6 |  | Dropping Test | The microphone unit must be subjected to a procedure that after dropping to a slippery marble floor for 5 times from a 1-meter-high without package. |
|  | 5.7 |  | ESD Test | The microphone under test must be discharged between each ESD exposure (contact : ±4KV, air: ±4KV) There is no interference in operation after 10 times exposure. |
| 6 | ## Environmental Condition: |  |  |  |
|  | 6.1 |  | ##### Storage condition | -40℃~+70℃ R.H. less than 90% |
|  | 6.2 |  | Operation condition | -40℃~+110℃ R.H. less than 90% |
|  | 6.3 |  | Arbitration condition | Temperature ：20℃±1℃ Relative humidity：63%~67% Air pressure ：86~106Kpa |
| 7 | Notices: |  |  |  |
|  | 7.1 | Always Avoid bringing pinholes on the soldering terminal during the operation to the omi-directional microphones. |  |  |
|  | 7.2 | Operators, the solder fixtures and the soldering irons must be statically grounded under each soldering process. |  |  |
|  | 7.3 | All the soldering procedures upon microphones must be completed in a metallic device, the temperature of the soldering irons must be limited as 320℃±10℃. Soldering time should not exceed 2 Seconds. |  |  |

PAGE 5/5

| 8 | Packing Specification: |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Drawing(Unit: mm) | Qty (pcs.) | Material | Marking |
|  | Packing |  | 100 | Paper |  |
|  | Middle Box |  | 10×100 | Paper |  |
|  | Outer Box |  | 20×1000 | Paper | Particular for Customer’s P.O |